# Load Utilization Framework

Space-Based Metric for 53' Step Deck Trailers

# 1. Overview

This framework defines how to measure load utilization for COT shipments. The primary metric is Space Utilization - the percentage of trailer capacity consumed by loaded products, accounting for both horizontal (length) and vertical (stacking) efficiency.

Key principles:

- Each position on the trailer consumes length based on the longest product in that position

- Full stacks receive full credit; partial stacks are penalized proportionally

- Mixed products can share a position and combine to fill vertical space

Figure 1: Examples of fully loaded COT trailers showing stacked products

# 2. Trailer Specifications

## 2.1 Trailer Types

COT primarily uses 53' step deck trailers for shipping. Alternative trailer types include:

- Step Deck (Primary): Two-level loading surface with 10' upper deck and 43' lower deck. Used for most USA, UTA, and DUMP shipments.

- Flatbed: Single-level 53' loading surface. May be used when step deck is unavailable or for specific product configurations.

- Wedge Trailer: Specialized trailer automatically used for Equipment (HDEQ) category units. Features angled design for loading heavy equipment.

Figure 2: 53' Step Deck Trailer Specifications

Figure 3: Standard 53' Flatbed Trailer

Figure 4: Wedge Trailer (used for Equipment category)

## 2.2 Step Deck Loading Zones

The step deck trailer has two distinct loading zones with physical constraints that must be considered:

| Zone | Length | Product Constraint | Notes |
| --- | --- | --- | --- |
| Upper Deck | 10' | Length ≤ 10' ONLY | Smaller products only |
| Lower Deck | 43' | All products | Main loading area |
| Total | 53' | — | Combined for utilization calculation |

# 3. Physical Capacity Check

Before calculating utilization percentage, verify the load physically fits on the trailer.

## 3.1 Capacity Check Process

- Classify each product: Length ≤ 10' can use either deck; Length > 10' is lower deck only

- Calculate lower deck requirement: Sum of (Positions × Length) for all products that MUST go on lower deck

- Check lower deck fit: If lower deck requirement > 43', load exceeds capacity

- Allocate remaining space: Small products (≤10') can overflow to upper deck if lower deck is full

## 3.2 Positions Available by Product Length

| Product Length | Lower Deck (43') | Upper Deck (10') | Max Positions |
| --- | --- | --- | --- |
| 7' | 6 positions | 1 position | 7 |
| 10' | 4 positions | 1 position | 5 |
| 12' | 3 positions | — | 3 |
| 16' | 2 positions | — | 2 |
| 20' | 2 positions | — | 2 |
| 22' | 1 position | — | 1 |
| 25'+ | 1 position | — | 1 |

# 4. Utilization Calculation

## 4.1 Position Credit Calculation

Each position's credit depends on how fully the vertical stack is utilized:

Position Credit = (Vertical Fill %) × (Length / 53')

Where:

- Vertical Fill % = Units in Position / Max Stack Height for that position

- Length = Length with Tongue of the longest product in the position

## 4.2 Full Stack vs. Partial Stack

- Full stack (100% fill): Full credit → Length / 53'

- Partial stack: Penalized → (Units / Max Stack) × (Length / 53')

## 4.3 Mixed Product Positions

Different products can share a position to maximize vertical fill:

- Combine vertical fill percentages from each product

- Use the longest product's length as the position footprint

- Cap total vertical fill at 100%

## 4.4 Total Load Utilization

Load Utilization = Σ (Position Credits)

# 5. Grading Scale

Load utilization grades based on total credit earned:

| Grade | Utilization | Description | Action |
| --- | --- | --- | --- |
| A | ≥ 85% | Excellent | Optimal load |
| B | 70% - 84% | Good | Acceptable |
| C | 55% - 69% | Fair | Review for consolidation |
| D | 40% - 54% | Poor | Consolidation needed |
| F | < 40% | Failing | Hold for more orders |

# 6. Worked Examples

## Example 1: Single Product with Partial Stack

Load: 17× 5X8GW2K (Length: 7', Max Stack: 6)

Capacity Check: 7' products can use upper deck. Need 3 positions × 7' = 21'. Lower deck (43') + upper deck (10') = 53' available. ✓ FITS

| Position | Units | Fill % | Length | Credit |
| --- | --- | --- | --- | --- |
| 1 | 6 | 6/6 = 100% | 7' | 7' × 100% = 7.0' |
| 2 | 6 | 6/6 = 100% | 7' | 7' × 100% = 7.0' |
| 3 (partial) | 5 | 5/6 = 83% | 7' | 7' × 83% = 5.8' |
| TOTAL | 17 |  |  | 19.8' |

Utilization = 19.8' / 53' = 37.4% → Grade F

## Example 2: Mixed Products Completing a Stack

Load: 15× 5X8GW2K (7', stack 6) + 1× 6X12DUMP (16', stack 3)

Capacity Check: DUMP at 16' requires lower deck. SA at 7' can use either. Total lower deck need: 16' (mixed position) + 14' (2× SA stacks) = 30' < 43'. ✓ FITS

The 15 SAs fill 2 full stacks (12 units) with 3 remaining. The DUMP can stack with the partial SA stack since 3/6 + 1/3 ≈ 83% vertical fill. Use DUMP's longer length (16') for that position.

| Position | Contents | Fill % | Length | Credit |
| --- | --- | --- | --- | --- |
| 1 | 6× SA | 6/6 = 100% | 7' | 7.0' |
| 2 | 6× SA | 6/6 = 100% | 7' | 7.0' |
| 3 (mixed) | 3× SA + 1× DUMP | 3/6 + 1/3 = 83% | 16' | 16' × 83% = 13.3' |
| TOTAL | 16 units |  |  | 27.3' |

Utilization = 27.3' / 53' = 51.5% → Grade D

## Example 3: Perfect Mixed Stack (Full Vertical Fill)

Load: 15× 5X8GW2K (7', stack 6) + 2× 6X12DUMP (16', stack 3)

Capacity Check: Same logic as Example 2. ✓ FITS

Same as above, but with 2 DUMPs. Position 3 now has 3 SAs (3/6 = 50%) + 2 DUMPs (2/3 = 67%). Combined = 117%, but we cap at 100%.

| Position | Contents | Fill % | Length | Credit |
| --- | --- | --- | --- | --- |
| 1 | 6× SA | 100% | 7' | 7.0' |
| 2 | 6× SA | 100% | 7' | 7.0' |
| 3 (mixed) | 3× SA + 2× DUMP | 50% + 67% → 100% | 16' | 16' × 100% = 16.0' |
| TOTAL | 17 units | All 100% |  | 30.0' |

Utilization = 30.0' / 53' = 56.6% → Grade C

Key insight: Adding 1 more DUMP improved utilization from 51.5% to 56.6% by completing the vertical fill on position 3.

## Example 4: Large Products Exceeding Physical Capacity

Load: 11× 7X16GW2BRK (20', stack 5)

Capacity Check: 20' products can ONLY go on lower deck (43'). Position 1: 20' → 23' remaining. Position 2: 20' → 3' remaining. Position 3: Needs 20' but only 3' available, and 20' > 10' so cannot use upper deck. ✗ EXCEEDS CAPACITY

| Position | Units | Fill % | Length | Credit |
| --- | --- | --- | --- | --- |
| 1 | 5 | 5/5 = 100% | 20' | 20.0' |
| 2 | 5 | 5/5 = 100% | 20' | 20.0' |
| 3 (won't fit) | 1 | — | 20' | N/A |
| MAX FIT | 10 |  |  | 40.0' |

Maximum capacity: 10 units (2 full stacks)

Utilization at max: 40.0' / 53' = 75.5% → Grade B

Key insight: Even though 75.5% < 100%, this load is at physical capacity due to deck constraints. The 11th unit must go on a second truck. This is why capacity check must happen BEFORE utilization calculation.

## Example 5: Well-Optimized Mixed Load

Load: 24× 5X8GW2K (7', stack 6) + 10× 6X12GW (16', stack 5)

Capacity Check: 6X12 at 16' requires lower deck: 2 positions × 16' = 32'. 5X8 at 7' needs 4 positions × 7' = 28'. Total = 60' > 53'. ✗ EXCEEDS CAPACITY

This load would need to be split across two trucks.

# 7. Reference: Common Product Parameters

Key products from the Master Load Building Cheat Sheet:

| Category | Dimension | Length | Max Stack |
| --- | --- | --- | --- |
| USA | 5X8 | 7' | 6 |
| USA | 5X8 (HS) | 12' | 3-5 |
| USA | 6X12 | 16' | 5 |
| UTA | 7X16 | 20' | 5 |
| UTA | 7X18 | 22' | 5 |
| HDEQ | 7X20 | 25' | 5 |
| HDEQ | 8.5X20 | 27' | 3-5 |
| DUMP | 6X12 | 16' | 3 |
| DUMP | 7X14 | 18' | 3 |

# 8. Quick Reference Summary

## 8.1 Process Flow

- Capacity Check: Verify load fits within deck constraints (43' lower + 10' upper for ≤10' products)

- Calculate Position Credits: (Vertical Fill %) × (Length / 53') for each position

- Sum Credits: Total utilization = Σ(Position Credits)

- Assign Grade: A (≥85%), B (70-84%), C (55-69%), D (40-54%), F (<40%)

## 8.2 Key Formulas

| Component | Formula |
| --- | --- |
| Lower Deck Positions | FLOOR(43' / Product Length) |
| Upper Deck Positions | FLOOR(10' / Product Length) — only if Length ≤ 10' |
| Full Stack Credit | Length / 53' |
| Partial Stack Credit | (Units / Max Stack) × (Length / 53') |
| Mixed Position Fill % | Σ(Units / Max Stack) for each product, cap at 100% |
| Mixed Position Length | Longest product's Length with Tongue |
| Load Utilization | Σ(Position Credits) — target ≥ 85% |